# AD-HTC Integrated Biorefinery Dashboard

Technical Report

MEG 315 — Applied Thermodynamics
Group 28
Department of Mechanical Engineering
University of Lagos
February 2026

# Table of Contents

1. Introduction

2. Theory of the AD-HTC Combined Cycle

3. System Architecture

4. Technology Stack

5. API Endpoints

6. Database Schema

7. Feedstock Database

8. Otto Cycle Analysis

9. Rankine Cycle Analysis

10. Heat Integration

11. Mass & Energy Balance

12. How to Run

13. Conclusion

14. References

# 1. Introduction

This report documents the design, implementation, and analysis of an AD-HTC integrated biorefinery dashboard developed as part of MEG 315 — Applied Thermodynamics at the University of Lagos. The dashboard simulates the thermodynamic behaviour of a combined Anaerobic Digestion (AD) and Hydrothermal Carbonization (HTC) system, including Otto cycle gas power generation, Rankine steam cycle waste heat recovery, and heat integration analysis.

The project features a full-stack web application with a FastAPI backend, SQLite database for persistent storage, and an interactive HTML/JavaScript frontend with real-time thermodynamic calculations and visualisations.

# 2. Theory of the AD-HTC Combined Cycle

## 2.1 Anaerobic Digestion (AD)

Anaerobic digestion is a biological process in which microorganisms break down organic matter in the absence of oxygen. Biomass feedstock (such as cow manure, food waste, or agricultural residues) is fed into a sealed reactor maintained at mesophilic conditions (35–40°C). The bacteria produce biogas, a mixture of methane (CH₄, 50–65%) and carbon dioxide (CO₂, 35–50%). The remaining material, called digestate, is a nutrient-rich slurry.

## 2.2 Hydrothermal Carbonization (HTC)

HTC is a thermochemical process that converts wet biomass into hydrochar — a carbon-rich solid fuel with properties similar to lignite coal. The process operates at 180–260°C under autogenous pressure (10–40 bar). HTC is particularly advantageous because it processes wet feedstocks directly, avoiding the energy cost of drying.

## 2.3 Gas Power Cycle (Otto)

The biogas produced by AD is used as fuel in an air-standard Otto cycle (internal combustion engine). The cycle consists of four processes:
• Process 1→2: Isentropic compression — T₂ = T₁ × r^(γ−1)
• Process 2→3: Constant-volume heat addition — Q_in = Cp × (T₃ − T₂)
• Process 3→4: Isentropic expansion — T₄ = T₃ / r^(γ−1)
• Process 4→1: Constant-volume heat rejection
The thermal efficiency is η = W_net / Q_in where W_net = W_turb − W_comp.

## 2.4 Steam Cycle (Rankine)

The waste heat from the gas turbine exhaust is recovered to generate steam via a Rankine-type heating loop:
• A→B: Pump raises liquid pressure from P_cond to P_boiler (60 bar)
• B→C: Boiler heats water to superheated steam using waste heat
• C→D: Steam transfers heat to HTC reactor (digestate carbonization)
• D→A: Steam condenses and returns to pump
Steam properties are obtained from a hardcoded P=6.0 MPa table with linear interpolation between data points.

## 2.5 System Integration

The key innovation is thermal self-sufficiency. Biogas fuels the gas engine, exhaust waste heat generates steam for HTC, and digestate from AD becomes the HTC feedstock. A heat integration analysis compares CHP waste heat against the combined thermal demands of the AD reactor (heating to 37°C) and the HTC reactor (heating to 180–260°C).

# 3. System Architecture

The application follows a client-server architecture:

• Frontend: Single HTML file (ad-htc-biomass-v2.html) with JavaScript, Chart.js, and Tailwind CSS
• Backend: FastAPI (Python) serving the dashboard and 3 API endpoints
• Database: SQLite with two tables (feedstocks, calculations)
• The HTML file works both standalone (local calculations) and with the FastAPI backend (API calls + database storage)

[Insert system architecture diagram here]

# 4. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Backend | Python / FastAPI | RESTful API for cycle calculations |
| Database | SQLite | Feedstock storage + calculation history |
| Validation | Pydantic | Input/output data models |
| Frontend | HTML / CSS / JS | Interactive dashboard UI |
| Charts | Chart.js | T-s, h-s, energy, mass, heat charts |
| Styling | Tailwind CSS | Responsive layout |
| Schematic | SVG | Animated process flow diagram |

# 5. API Endpoints

The FastAPI backend exposes the following RESTful endpoints:

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | / | Serve the dashboard HTML page |
| GET | /api/feedstocks | Return all 12 feedstocks from SQLite |
| POST | /api/analyze | Run cycle analysis, store result in DB |
| GET | /api/history | Return past calculation results |

The POST /api/analyze endpoint accepts a JSON body with all sidebar parameters (feedstock_key, feed_rate, T1, compression_ratio, T3, mass_flow_air, t_steam_C, p_cond, eta_turbine, eta_pump, htc_temp) and returns a structured JSON response containing otto{}, rankine{}, mass_balance{}, heat{}, and olr values.

# 6. Database Schema

The SQLite database (adhtc.db) contains two tables:

## 6.1 feedstocks

| Column | Type | Description |
| --- | --- | --- |
| id | INTEGER PK | Auto-increment primary key |
| key | TEXT UNIQUE | Identifier (e.g. cow_manure) |
| name | TEXT | Display name |
| ts | REAL | Total solids fraction |
| vs | REAL | Volatile solids fraction |
| biogas_yield | REAL | Biogas yield (m³/kg VS) |
| ch4 | REAL | Methane fraction |
| htc_yield | REAL | Hydrochar mass yield |
| htc_hhv | REAL | Hydrochar HHV (kJ/kg) |

## 6.2 calculations

| Column | Type | Description |
| --- | --- | --- |
| id | INTEGER PK | Auto-increment primary key |
| timestamp | TEXT | ISO 8601 timestamp |
| feedstock | TEXT | Feedstock key used |
| inputs | JSON | All input parameters |
| otto_results | JSON | Otto cycle outputs |
| rankine_results | JSON | Rankine cycle outputs |
| heat_balance | REAL | Heat surplus/deficit (kW) |
| biogas_vol | REAL | Biogas volume (m³/day) |
| elec_power | REAL | Electrical power (kW) |
| hydrochar | REAL | Hydrochar output (kg/day) |

# 7. Feedstock Database

The database is seeded with 12 real biomass feedstocks with published data:

| Feedstock | TS% | VS% | Biogas (m³/kg VS) | CH₄% | HTC Yield | HTC HHV |
| --- | --- | --- | --- | --- | --- | --- |
| Cow Manure | 18 | 80 | 0.280 | 60 | 48% | 14,500 |
| Food / Kitchen Waste | 25 | 90 | 0.550 | 62 | 45% | 19,800 |
| Sewage Sludge | 30 | 70 | 0.300 | 58 | 50% | 15,000 |
| Corn Stover | 88 | 85 | 0.338 | 55 | 55% | 21,500 |
| Rice Husk | 90 | 80 | 0.250 | 52 | 58% | 18,200 |
| Sugarcane Bagasse | 50 | 90 | 0.310 | 54 | 52% | 20,100 |
| Pig Manure | 20 | 82 | 0.320 | 62 | 46% | 15,200 |
| Wood Chips | 85 | 92 | 0.200 | 55 | 65% | 24,000 |
| Municipal Solid Waste | 60 | 75 | 0.400 | 58 | 50% | 17,500 |
| Microalgae | 10 | 85 | 0.450 | 65 | 40% | 22,000 |
| Cassava Peel | 88 | 86 | 0.420 | 58 | 48% | 19,200 |
| Palm EFB | 78 | 85 | 0.270 | 53 | 60% | 20,800 |

# 8. Otto Cycle Analysis

Sample calculation with default parameters:
• Intake temperature T₁ = 298 K
• Compression ratio r = 10
• Peak temperature T₃ = 1200 K
• Air mass flow ṁ = 50 kg/s

## 8.1 State Points

| Point | Process | T (K) | s (kJ/kg·K) |
| --- | --- | --- | --- |
| 1 | Intake | 298.0 | 1.000 |
| 2 | Compression | 748.5 | 1.000 |
| 3 | Combustion | 1200.0 | 1.542 |
| 4 | Exhaust | 477.7 | 1.542 |

## 8.2 Performance

• Compression work: W_comp = 452.8 kJ/kg
• Expansion work: W_turb = 829.2 kJ/kg
• Net work: W_net = 376.4 kJ/kg
• Heat input: Q_in = 518.3 kJ/kg
• Thermal efficiency: η = 72.6%
• Electrical power: 18819 kW

[Insert T-s diagram screenshot here]

# 9. Rankine Cycle Analysis

Steam cycle at P = 6.0 MPa (60 bar) with hardcoded steam table data.
• Steam temperature: 500°C
• Condenser pressure: 0.06 bar
• Turbine efficiency: 0.85
• Pump efficiency: 0.80

## 9.1 State Points

| Point | Description | h (kJ/kg) | s (kJ/kg·K) |
| --- | --- | --- | --- |
| A | Pump Inlet | 359.9 | 1.1450 |
| B | Pump Outlet | 367.4 | 1.1450 |
| C | Turbine Inlet | 3423.1 | 6.8826 |
| D | Turbine Exit | 2570.5 | 7.3016 |

## 9.2 Performance

• Pump work: w_pump = 5.99 kJ/kg
• Net work: w_net = 845.1 kJ/kg
• Boiler heat: q_boiler = 3055.7 kJ/kg
• Thermal efficiency: η = 27.7%

[Insert h-s Mollier diagram screenshot here]

# 10. Heat Integration

The heat integration analysis compares available CHP waste heat against the thermal demands of the AD and HTC reactors:

• CHP waste heat = ṁ_air × Cp × (T₄ − T₁) × 0.45
• AD demand = (feed kg/s) × 4180 × (37°C − ambient) / 1000
• HTC demand = (digestate kg/s) × 4000 × (T_HTC − 37°C) / 1000
• Heat balance = CHP output − AD demand − HTC demand

If the balance is positive, the system is self-sufficient (SURPLUS). If negative, external heating is required (DEFICIT).

API test result: Heat balance = +4555.89 kW (SURPLUS) for Cow Manure at 10 t/day.

# 11. Mass & Energy Balance

Sample: Cow Manure at 10 tonnes/day

| Output | Value | Unit |
| --- | --- | --- |
| Feed input | 10000 | kg/day |
| Volatile solids | 1440 | kg/day |
| Biogas volume | 403 | m³/day |
| Methane volume | 242 | m³/day |
| Biogas mass | 484 | kg/day |
| Dry digestate | 576 | kg/day |
| Hydrochar | 276 | kg/day |

# 12. How to Run

Option A — Full-stack (with API + database):
  1. pip install -r requirements.txt
  2. python main.py
  3. Open http://localhost:8000
  4. API docs at http://localhost:8000/docs

Option B — Standalone (no installation):
  1. Open ad-htc-biomass-v2.html in any web browser
  2. All calculations run locally in JavaScript

# 13. Conclusion

This project demonstrates a complete AD-HTC biorefinery simulation with full-stack web development. Key achievements:

• FastAPI backend with 4 RESTful API endpoints
• SQLite database storing 12 feedstocks and calculation history
• Otto cycle analysis with η = 72.6%
• Rankine steam cycle with η = 27.7%
• Heat integration showing system self-sufficiency
• Interactive dashboard with Chart.js visualisations
• Zero-installation standalone mode also available

The project covers key thermodynamic concepts including the First Law, Second Law, cycle analysis, and heat integration, while demonstrating modern software engineering practices (RESTful API, SQL, data validation).

# 14. References

[1] Potrč, S., Petrovič, A., Egieya, J. M., & Čuček, L. (2025). Valorization of biomass through anaerobic digestion and hydrothermal carbonization. Energies, 18(2), 334.

[2] Çengel, Y. A., & Boles, M. A. (2019). Thermodynamics: An Engineering Approach (9th ed.). McGraw-Hill.

[3] FastAPI Documentation. https://fastapi.tiangolo.com/

[4] SQLite Documentation. https://www.sqlite.org/docs.html

[5] Chart.js Documentation. https://www.chartjs.org/docs/